MATERIAL SAFETY DATA SHEET

DIBUTYL TIN OXIDE 98% FOR SYNTHESIS

(D.B.T.O.)

MSDS CAS: 818-08-6

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: DIBUTYL TIN OXIDE 98% FOR SYNTHESIS

CAS#: 818-08-6

Chemical Name: DIBUTYL TIN OXIDE

Synonyms: Dibutyloxotin

Chemical Formula: C8H18OSn

Brand : OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| DIBUTYL TIN OXIDE | 818-08-6 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45

Classification : T; R25

Xn; R22

Xi; R36/38

N; R51-53 
Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards : Eye irritation - Category 2A - Warning (CLP : Eye Irrit. 2) H319

Acute toxicity, Oral - Category 4 - Warning (CLP : Acute Tox. 4) H302

Skin irritation - Category 2 - Warning (CLP : Skin Corr. 2) H315

Environmental hazards : Hazardous to the aquatic environment - Chronic hazard - Category 2 (CLP :

Aquatic Chronic 2) H411

Other hazards: This substance/mixture contains no components considered to be either persistent, bioaccumulative and toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher.

Section 4: First Aid Measures

Eye Contact: Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

Skin Contact: Wash contaminated clothing before reuse. Wash with plenty of soap and water. If skin irritation occurs : Get medical advice. Specific treatment (see on this label).

Serious Skin Contact: Not available.

Inhalation: Assure fresh air breathing. Allow the victim to rest.

Serious Inhalation: Not available.

Ingestion: Obtain emergency medical attention. Call a POISON CENTER or doctor if you feel unwell. Rinse mouth. Do NOT induce vomiting.

Serious Ingestion: Not available.

Section 5: Fire and Explosion Data

Extinguishing media:

Suitable extinguishing media         : Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media     : Do not use a heavy water stream.

Surrounding fires                            : Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture:

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters:

Protection against fire                     : Do not enter fire area without proper protective equipment,             including respiratory protection.

Special procedures                           : Exercise caution when fighting any chemical fire. Avoid (reject)                            fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

For emergency responders        :Equip cleanup crew with proper protection.

:Ventilate area.

For non-emergency personnel  :Evacuate unnecessary personnel.

Environmental precautions

Environmental precautions       :Prevent entry to sewers and public waters. Notify authorities if product           enters sewers or public waters.

Methods and material for containment and cleaning up:

Clean up methods                       : Soak up spills with inert solids, such as clay or diatomaceous earth as           soon as possible. Collect spillage. Store away from other materials.

Section 7: Handling and Storage

Precautions for safe handling:

Handling: Do not eat, drink or smoke when using this product. Wash thoroughly after handling. Wash thoroughly after handling.

Technical protective measures: Provide good ventilation in process area to prevent formation of vapour.

Conditions for safe storage, including any incompatibilities:

Storage: Keep only in the original container in a cool, well ventilated place. Keep container closed when not in use.

Storage - away from: Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Section 8: Exposure Controls/Personal Protection

Exposure controls:

Personal protection: Avoid all unnecessary exposure.

• Respiratory protection: Wear approved mask.

• Hand protection: Wear protective gloves.

• Skin protection: Wear suitable protective clothing.

• Eye protection: Chemical goggles or safety glasses.

• Others: When using, do not eat, drink or smoke.

Control parameters:

Occupational Exposure Limits: No data available.

Section 9: Physical and Chemical Properties

Physical state and appearance	: Powder

Odor						: Not available.

Taste						: Not available.

Molecular Weight			: Not available.

Color						: White.

pH (1% soln/water)			: Not available.

Boiling Point				: 161.9 °C

Section 9: Physical and Chemical Properties (Continued)

Melting Point				: : >= 300 °C.

Critical Temperature			: Not available.

Specific Gravity				: Not available.

Vapor Pressure				: Not available.

Vapor Density				: Not available.

Volatility					: Not available.

Odor Threshold				: Not available.

Water/Oil Dist. Coeff.			: Not available.

Ionicity (in Water)			: Not available.

Dispersion Properties			: Not available.

Solubility					: Not available.

Section 10: Stability and Reactivity Data

Stability: The product is stable.

Instability Temperature: Not available.

Conditions to avoid:  Direct sunlight. Extremely high or low temperatures.

Incompatibility with various substances: Not available.

Materials to avoid:  Strong acids. Strong bases.

Special Remarks on Reactivity: Not available.

Special Remarks on Corrosivity: Not available.

Hazardous decomposition products:  Fumes. Carbon monoxide. Carbon dioxide.

Polymerization: No.

Section 11: Toxicological Information

Acute toxicity

• Inhalation : Based on available data, the classification criteria are not met.

• Dermal : Based on available data, the classification criteria are not met.

• Ingestion : Harmful if swallowed.

Corrosion : Based on available data, the classification criteria are not met.

Irritation : Causes serious eye irritation.

Causes skin irritation.

Sensitization : Based on available data, the classification criteria are not met.

Mutagenicity : Based on available data, the classification criteria are not met.

Carcinogenicity : Based on available data, the classification criteria are not met.

Toxic for reproduction : Based on available data, the classification criteria are not met.

STOT-single exposure : Based on available data, the classification criteria are not met.

STOT-repeated exposure : Based on available data, the classification criteria are not met.

Aspiration hazard : Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity information:  Toxic to aquatic life with long lasting effects.

BOD5 and COD: Not available.

Persistence - degradability:  May cause long-term adverse effects in the environment.

Bioaccumulative potential:  Not established.

Toxicity of the Products of Biodegradation: Not available.

Special Remarks on the Products of Biodegradation: Not available.

Results of PBT and vPvB assessment:  The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Environmental precautions:  Avoid release to the environment.

Section 13: Disposal Considerations

Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations. Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name		: ORGANOTIN COMPOUND, SOLID, N.O.S.
UN N°				: 3146

ADR – Class			: 6.1

Packaging group ADR/RID	: III

Sea transport (IMDG) [English only]

Proper shipping name		: ORGANOTIN COMPOUND, SOLID, N.O.S.
UN N°				: 3146

IMO-IMDG - Class or division: 6.1

IMO-IMDG - Packing group	: II

Air transport (ICAO-IATA) [English only]

Proper shipping name		: ORGANOTIN COMPOUND, SOLID, N.O.S.
UN N°				: 3146

IATA - Class or division	: 6.1

IATA - Packing group		: II

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture:

Safety, health and environmental      : Ensure all national/local regulations are observed. regulations/legislation specific for

the substance or mixture.

REACH Restrictions - Annex XVII   : The components of this product are not subject to restrictions.

REACH Authorisation - Annex XIV : The components of this product are not subject to authorization.

Section 15: Other Regulatory Information (Continued)

Chemical Safety Assessment:

Chemical Safety Assessment               : It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.