MATERIAL SAFETY DATA SHEET

Meta-DICHLOROBENZENE 98%

Extra Pure (M.D.C.B) (1,3 Dichlorobenzene)

MSDS CAS: 541-73-1

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: Meta-DICHLOROBENZENE

CAS#:   541-73-1

Synonym: 1,3 Dichlorobenzene

Chemical Name: 	Meta-Dichlorobenzene

Chemical Formula:  C6H4Cl2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Meta-Dichlorobenzene | 541-73-1 | 100 |

Section 3: Hazards Identification

Risk advice to man and the environment:

Harmful if swallowed. Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Section 4: First Aid Measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing give artificial respiration Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 5: Fire and Explosion Data

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special protective equipment for fire-fighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Section 6: Accidental Release Measures

Personal precautions:

Use personal protective equipment. Avoid breathing vapors, mist or gas. Ensure adequate ventilation.

Section 6: Accidental Release Measures (Continued)

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Discharge into the environment must be avoided.

Methods for cleaning up:

Soak up with inert absorbent material and dispose of as hazardous waste. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Handling:

Avoid inhalation of vapour or mist. Keep away from sources of ignition - No smoking.

Take measures to prevent the buildup of electrostatic charge.

Storage:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Section 8: Exposure Controls/Personal Protection

Personal protective equipment

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multipurpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Hand protection:

The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it. Handle with gloves.

Eye protection: Face shield and safety glasses.

Skin and body protection:

Choose body protection according to the amount and concentration of the dangerous substance at the work place.

Section 8: Exposure Controls/Personal Protection (Continued)

Hygiene measures:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Section 9: Physical and Chemical Properties

Physical state					: Liquid

Molecular mass					: 147 g/mol

Colour						: Clear Colorless.

Odour						: pleasant odor.

Odour threshold					: No data available

pH							: No data available

Relative evaporation rate (butylacetate=1): No data available

Melting point					: -25 - -22 °C

Freezing point					: No data available

Boiling point					: 172 - 173 °C

Flash point						: 67 °C

Auto-ignition temperature			: No data available

Decomposition temperature			: No data available

Flammability (solid, gas)			: No data available

Vapour pressure					: No data available

Relative vapour density at 20 °C		: No data available

Relative density					: No data available

Density						: 1.288 g/cm³

Solubility						: No data available

Log Pow						: 5

Viscosity, kinematic				: No data available

Viscosity, dynamic				: No data available

Explosive properties				: No data available

Oxidising properties				: No data available

Explosive limits					: No data available

Section 10: Stability and Reactivity Data

Storage stability:

Stable under recommended storage conditions.

Materials to avoid:

Strong oxidizing agents.

Hazardous decomposition products:

Hazardous decomposition products formed under fire conditions. - Carbon oxides, Hydrogen chloride gas.

Section 11: Toxicological Information

Acute toxicity:

LD50 Intraperitoneal - mouse - 1.062 mg/kg

Irritation and corrosion: No data available.

Sensitization: No data available.

Chronic exposure:

IARC: 3 - Group 3: Not classifiable as to its carcinogenicity to humans (1,3-Dichlorobenzene).

Signs and Symptoms of Exposure:

To the best of our knowledge, the chemical, physical, and toxicological properties have not been thoroughly investigated.

Potential Health Effects

Inhalation May be harmful if inhaled. May cause respiratory tract irritation.

Skin May be harmful if absorbed through skin. May cause skin irritation.

Eyes May cause eye irritation.

Ingestion Harmful if swallowed.

Section 12: Ecological Information

Elimination information (persistence and degradability):

Bioaccumulation Pimephales promelas (fathead minnow) - 32 d Bioconcentration factor (BCF): 97

Section 12: Ecological Information (Continued)

Ecotoxicity effects:

Toxicity to fish LC50 - Pimephales promelas (fathead minnow) - 7,8 mg/l - 96 h Toxicity to daphnia and other aquatic invertebrates. LC50 - Daphnia magna (Water flea) - 1,7 mg/l - 48 h

Further information on ecology:

Toxic to aquatic or ganisms, may cause long-term adverse effects in the aquatic environment.

Section 13: Disposal Considerations

Product:

This combustible material may be burned in a chemical incinerator equipped with an afterburner and scrubber. Observe all federal, state, and local environmental regulations. Contact a licensed professional waste disposal service to dispose of this material.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S.

UN N°: 3082

H.I. nr: 90

ADR - Class: 9

Labelling - Transport: 9 : Miscellaneous dangerous substances and articles.

ADR - Group: III

Sea transport (IMDG) [English only]

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S.

UN N°: 3082

IMO-IMDG - Class or division: 9 : Miscellaneous dangerous substances and articles.

IMO-IMDG - Packing group: III

Section 14: Transport Information (Continued)

Air transport (ICAO-IATA) [English only]

Proper shipping name: ENVIRONMENTALLY HAZARDOUS SUBSTANCE, LIQUID, N.O.S.

UN N°: 3082

IATA - Class or division: 9 : Miscellaneous dangerous substances and articles.

IATA - Packing group: III

Section 15: Other Regulatory Information

Labelling according to EC Directives

EC Label

Hazard symbols

Xn Harmful

N Dangerous for the environment.

R-phrase(s):

R22 Harmful if swallowed.

R51/53 Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

S-phrase(s):

S61 Avoid release to the environment. Refer to special instructions/ Safety data sheets.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.