MATERIAL SAFETY DATA SHEET

DICYCLOHEXYLAMINE NITRITE

MSDS CAS: 3129-91-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: DICYCLOHEXYLAMINE NITRITE

CAS#:   3129-91-7

Synonym: Dicyclohexylammonium nitrite

Chemical Name: 	Dicyclohexylamine Nitrite

Chemical Formula:  C12H23N·HNO2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:      OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Dicyclohexylamine Nitrite | 3129-91-7 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008 [EU-GHS/CLP]

Flammable solids (Category 2)

Acute toxicity, Oral (Category 4)

Acute toxicity, Inhalation (Category 4)

Classification according to EU Directives 67/548/EEC or 1999/45/EC

Harmful by inhalation and if swallowed.

Label elements

Labelling according Regulation (EC) No 1272/2008 [CLP]

Hazard statement(s):

H228 Flammable solid.

H302 + H332 Harmful if swallowed or if inhaled.

Precautionary statement(s):

P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking.

Supplemental Hazard Statements: None.

According to European Directive 67/548/EEC as amended.

R-phrase(s):

R20/22 Harmful by inhalation and if swallowed.

S-phrase(s):

S15 Keep away from heat.

S41 In case of fire and/or explosion do not breathe fumes.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Consult a physician.

Section 4: First Aid Measures (Continued)

In case of eye contact:

Flush eyes with water as a precaution.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed: No data available

Indication of any immediate medical attention and special treatment needed:

No data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:

Carbon oxides, nitrogen oxides (NOx).

Advice for firefighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Further information:

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Use personal protective equipment. Avoid dust formation. Avoid breathing vapors, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas. Avoid breathing dust.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Section 6: Accidental Release Measures (Continued)

Methods and materials for containment and cleaning up:

Sweep up and shovel. Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations (see section 13). Keep in suitable, closed containers for disposal. Contain spillage, pick up with an electrically protected vacuum cleaner or by wet-brushing and transfer to a container for disposal according to local regulations.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid formation of dust and aerosols.

Provide appropriate exhaust ventilation at places where dust is formed. Keep away from sources of ignition - No smoking. Take measures to prevent the build-up of electrostatic charge.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Recommended storage temperature: 2 - 8 °C

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Personal protective equipment

Eye/face protection:

Safety glasses with side-shields conforming to EN166 Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Section 8: Exposure Controls/Personal Protection (Continued)

Body Protection: Complete suit protecting against chemicals, Flame retardant antistatic protective clothing, and the type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 9: Physical and Chemical Properties

Appearance Form		: Powder.

Colour				: Beige.

Molecular Weight		: 228.33 g/mole

Odour				: No data available.

Odour				: Threshold no data available.

pH					: No data available.

Melting point			: 182 - 183 °C

Boiling point			: No data available.

Flash point				: No data available.

Evaporation rate			: No data available.

Flammability (solid, gas)	: The substance or mixture is a flammable solid with the category 2.

Explosive limits			: No data available.

Vapour pressure			: 4,4 hPa at 25 °C

Vapour density			: No data available.

Relative density			: No data available.

Water solubility			: No data available.

Partition coefficient: noctanol/water: No data available.

Auto-ignition temperature	: No data available.

Decomposition temperature	: No data available.

Viscosity				: No data available.

Explosive properties		: No data available.

Oxidizing properties		: No data available.

Section 10: Stability and Reactivity Data

Reactivity: No data available.

Chemical stability: No data available.

Possibility of hazardous reactions: No data available.

Conditions to avoid:

Heat, flames and sparks. Extremes of temperature and direct sunlight.

Incompatible materials:

Strong oxidizing agents,

Hazardous decomposition products:

Other decomposition products - no data available.

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity

LD50 Oral - rat - 330 mg/kg

Remarks: Liver: Other changes. Blood: Normocytic anemia.

Skin corrosion/irritation:  No data available.

Serious eye damage/eye irritation:  No data available.

Respiratory or skin sensitization:  No data available.

Germ cell mutagenicity:  No data available.

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Reproductive toxicity: No data available

Specific target organ toxicity - single exposure: No data available

Section 11: Toxicological Information

Specific target organ toxicity - repeated exposure: No data available

Aspiration hazard: No data available

Potential health effects

Inhalation Harmful if inhaled. May cause respiratory tract irritation.

Ingestion Harmful if swallowed.

Skin May be harmful if absorbed through skin. May cause skin irritation.

Eyes May cause eye irritation.

Section 12: Ecological Information

Toxicity: No data available.

Persistence and degradability: No data available.

Bioaccumulative potential: No data available.

Mobility in soil: No data available.

Results of PBT and vPvB assessment: No data available.

Other adverse effects: No data available.

Section 13: Disposal Considerations

Waste treatment methods

Product:

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: DICYCLOHEXYLAMMONIUM NITRITE

UN N°: 2687

H.I. nr: -

ADR - Class -Transport: 4.1

ADR - Group: III

Sea transport (IMDG) [English only]

Proper shipping name: DICYCLOHEXYLAMMONIUM NITRITE

UN N°: 2687

IMO-IMDG-Class or division: 4.1

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: DICYCLOHEXYLAMMONIUM NITRITE

UN N°: 2687

IATA - Class or division: 4.1

IATA - Packing group: III

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture

No data available.

Chemical Safety Assessment: No data available.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine Chem LLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.