MATERIAL SAFETY DATA SHEET

ETHYLENE THIOUREA

MSDS CAS: 96-45-7

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: ETHYLENE THIOUREA

CAS#:96-45-7

Synonym:2-Thioxoimidazolidine, 2-Imidazolidinethione

Chemical Name: Ethylene Thiourea

Chemical Formula:C3H6N2S

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Ethylene Thiourea | 96-45-7 | 100 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Acute toxicity, Oral (Category 4), H302

Carcinogenicity (Category 2), H351

Reproductive toxicity (Category 1B), H360D

Specific target organ toxicity - repeated exposure (Category 1), H372

Label elements

Labelling according Regulation (EC) No 1272/2008

Hazard statement(s):

H302 Harmful if swallowed.

H351 Suspected of causing cancer.

H360D May damage the unborn child.

H372 Causes damage to organs through prolonged or repeated exposure.

Precautionary statement(s):

P201 Obtain special instructions before use.

P281 Use personal protective equipment as required.

P308 + P313 IF exposed or concerned: Get medical advice/ attention.

Supplemental Hazard Statements:None.

Other hazards:

This substance/mixture contains no components considered to be either persistent, bioaccumulative or toxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Take victim immediately to hospital. Consult a physician.

Section 4: First Aid Measures (Continued)

In case of eye contact:Flush eyes with water as a precaution.

If swallowed:

Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Most important symptoms and effects, both acute and delayed:

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

Indication of any immediate medical attention and special treatment needed:

No data available.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:

Carbon oxides, Nitrogen oxides (NOx), Sulphur oxides.

Advice for firefighters:

Wear self-contained breathing apparatus for firefighting if necessary.

Further information:No data available.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Use personal protective equipment. Avoid dust formation. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Evacuate personnel to safe areas. Avoid breathing dust.

For personal protection see section 8.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Section 6: Accidental Release Measures (Continued)

Methods and materials for containment and cleaning up:

Pick up and arrange disposal without creating dust. Sweep up and shovel. Keep in suitable, closed containers for disposal.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Avoid exposure - obtain special instructions before use. Provide appropriate exhaust ventilation at places where dust is formed.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place.

Specific end use(s):

Apart from the uses mentioned in section 1.2 no other specific uses are stipulated.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Personal protective equipment

Eye/face protection:

Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices.Wash and dry hands.

Section 8: Exposure Controls/Personal Protection (Continued)

Body Protection:

Complete suit protecting against chemicals, the type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face particle respirator type N100 (US) or type P3 (EN 143) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Control of environmental exposure:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Section 9: Physical and Chemical Properties

Appearance Form		: powder

Colour				: white

Molecular Weight		: 102.16 g/mole

Odour				: No data available

Odour Threshold			: No data available

pH					: No data available

Melting point			: 196 - 200 °C

Boiling point			: 347 °C

Flash point				: No data available

Evaporation rate			: No data available

Flammability (solid, gas)	: No data available

Explosive limits			: No data available

Vapour pressure			: No data available

Vapour density			: No data available

Relative density			: 0,4512 g/cm3 at 20 °C

Water solubility			: 27,4 g/l at 20 °C at 6,88 hPa - completely soluble

Partition coefficient: noctanol/water log Pow: -0,67 at 20 °C

Auto-ignition temperature	: No data available

Decomposition temperature	: No data available

Section 9: Physical and Chemical Properties (Continued)

Viscosity				: No data available

Explosive properties		: No data available

Oxidizing properties		: No data available

Other safety information

Solubility in other		: Ethanol - slightly soluble

Solvents				: Benzene - insoluble

Surface tension			: ca.65,7mN/m at 23 °C

Section 10: Stability and Reactivity Data

Reactivity:No data available.

Chemical stability:

Stable under recommended storage conditions.

Possibility of hazardous reactions:No data available

Conditions to avoid:No data available

Incompatible materials:No data available

Hazardous decomposition products:

Other decomposition products - No data available

In the event of fire: see section 5

Section 11: Toxicological Information

Acute toxicity:LD50 Oral - Rat - 1.832 mg/kg

Skin corrosion/irritation

Skin - Rabbit

Result: No skin irritation

(OECD Test Guideline 404)

Section 11: Toxicological Information (Continued)

Serious eye damage/eye irritation

Eyes - Rabbit

Result: No eye irritation

(OECD Test Guideline 405)

Respiratory or skin sensitisation

- Mouse

Did not cause sensitisation on laboratory animals.

Germ cell mutagenicity

in vitro assay

S. typhimurium

Result: negative

Specific target organ toxicity - single exposure:No data available.

Specific target organ toxicity - repeated exposure:No data available.

Aspiration hazard:No data available.

Section 12: Ecological Information

Toxicity:

Toxicity to fish semi-static test LC50 - Poeciliareticulata (guppy) - 7.500 mg/l - 96 h

(OECD Test Guideline 203)

Toxicity to algae Growth inhibition EC50 - Chlorella pyrenoidosa - 6.600 mg/l - 96 h

Persistence and degradability:

Biodegradability aerobic - Exposure time 28 d Result: 0 % - Not readily biodegradable.

(OECD Test Guideline 301F)

Bioaccumulative potential:No data available

Mobility in soil:No data available

Results of PBT and vPvB assessment:

This substance/mixture contains no components considered to be either persistent, bioaccumulativeandtoxic (PBT), or very persistent and very bioaccumulative (vPvB) at levels of 0.1% or higher.

Section 13: Disposal Considerations

Waste treatment methods

Product:

Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber.

Contaminated packaging:

Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

General information: Not regulated.

Sea transport (IMDG) [English only]

General information: Not regulated.

Air transport (ICAO-IATA) [English only]

General information: Not regulated.

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

Authorisations and/or restrictions on use

2-Imidazolidinethione CAS-No.: 96-45-7

REACH - Candidate List of Substances of Very High Concern for Authorisation (Article 59).

Toxic for reproduction (article 57c)

Section 15: Other Regulatory Information (Continued)

2-Imidazolidinethione CAS-No.: 96-45-7

REACH - Candidate List of Substances of Very High Concern for Authorisation (Article 59).

Toxic for reproduction (article 57c)ED/121/2013

Chemical Safety Assessment:

For this product a chemical safety assessment was not carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.