MATERIAL SAFETY DATA SHEET

ETHYL ISOBUTYRATE 99%

MSDS CAS: 97-62-1

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name: ETHYL ISOBUTYRATE

CAS#:97-62-1

Synonym:Not Available.

Chemical Name: Ethyl Isobutyrate

Chemical Formula:C6H12O2

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Name | CAS # | % by Weight |
| --- | --- | --- |
| Ethyl Isobutyrate | 97-62-1 | 99% |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification according to Regulation (EC) No 1272/2008

Flammable liquids (Category 2), H225

Skin irritation (Category 2), H315

Eye irritation (Category 2), H319

Specific target organ toxicity - single exposure (Category 3), H335

Label elements

Labelling according Regulation (EC) No 1272/2008

Hazard statement(s):

H225 Highly flammable liquid and vapour.

H315 Causes skin irritation.

H319 Causes serious eye irritation.

H335 May cause respiratory irritation.

Precautionary statement(s):

P210 Keep away from heat/sparks/open flames/hot surfaces. - No smoking.

P261 Avoid breathing vapours.

P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Section 4: First Aid Measures

Description of first aid measures

General advice:

Consult a physician. Show this safety data sheet to the doctor in attendance.

If inhaled:

If breathed in, move person into fresh air. If not breathing, give artificial respiration. Consult a physician.

In case of skin contact:

Wash off with soap and plenty of water. Consult a physician.

In case of eye contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

If swallowed:

Do NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a physician.

Section 4: First Aid Measures (Continued)

Most important symptoms and effects, both acute and delayed:

The most important known symptoms and effects are described in the labelling (see section 2.2) and/or in section 11

Indication of any immediate medical attention and special treatment needed:

No data available

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media:

Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide.

Special hazards arising from the substance or mixture:Carbon oxides

Advice for firefighters:

Wear self contained breathing apparatus for fire fighting if necessary.

Further information:

Use water spray to cool unopened containers.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures:

Use personal protective equipment. Avoid breathing vapours, mist or gas. Ensure adequate ventilation. Remove all sources of ignition. Evacuate personnel to safe areas. Beware of vapours accumulating to form explosive concentrations. Vapours can accumulate in low areas.

Environmental precautions:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Methods and materials for containment and cleaning up:

Contain spillage, and then collect with an electrically protected vacuum cleaner or by wet-brushing and place in container for disposal according to local regulations.

Section 7: Handling and Storage

Precautions for safe handling:

Avoid contact with skin and eyes. Avoid inhalation of vapour or mist. Keep away from sources of ignition - No smoking. Take measures to prevent the build-up of electrostatic charge.

Conditions for safe storage, including any incompatibilities:

Store in cool place. Keep container tightly closed in a dry and well-ventilated place. Containers which are opened must be carefully resealed and kept upright to prevent leakage.

Section 8: Exposure Controls/Personal Protection

Control parameters

Components with workplace control parameters

Exposure controls

Appropriate engineering controls:

Handle in accordance with good industrial hygiene and safety practice. Wash hands before breaks and at the end of workday.

Personal protective equipment

Eye/face protection:

Face shield and safety glasses Use equipment for eye protection tested and approved under appropriate government standards such as NIOSH (US) or EN 166(EU).

Skin protection:

Handle with gloves. Gloves must be inspected prior to use. Use proper glove removal technique (without touching glove's outer surface) to avoid skin contact with this product. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practices.

Wash and dry hands.

Body Protection:

Impervious clothing, Flame retardant antistatic protective clothing, the type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace.

Respiratory protection:

Where risk assessment shows air-purifying respirators are appropriate use a full-face respirator with multi-purpose combination (US) or type ABEK (EN 14387) respirator cartridges as a backup to engineering controls. If the respirator is the sole means of protection, use a full-face supplied air respirator. Use respirators and components tested and approved under appropriate government standards such as NIOSH (US) or CEN (EU).

Section 8: Exposure Controls/Personal Protection (Continued)

Control of environmental exposure:

Prevent further leakage or spillage if safe to do so. Do not let product enter drains.

Section 9: Physical and Chemical Properties

Appearance Form			: Clear, Liquid

Colour					: Colourless.

Molecular Weight			: 116.16 g/mole

Odour					: No Data Available

Odour Threshold				: No Data Available

pH						: No Data Available

Melting Point				: No Data Available

Boiling Range				: 112 - 113 °C - Lit.

Flash Point					: 20 °C - Closed Cup

Evapouration Rate			: No Data Available

Flammability (Solid, Gas)		: No Data Available

Explosive Limits				: No Data Available

Vapour Pressure				: 53 Hpa At 33,8 °C

Vapour Density				: 4,01 - (Air = 1.0)

Relative Density				: 0,865 G/Cm3 At 25 °C

Water Solubility				: No Data Available

Partition Coefficient: Noctanol/Water: No Data Available

Auto-Ignition Temperature		: No Data Available

Decomposition Temperature		: No Data Available

Viscosity					: No Data Available

Explosive Properties			: No Data Available

Oxidizing Properties			: No Data Available

Relative Vapour Density		: 4,01 - (Air = 1.0)

Section 10: Stability and Reactivity Data

Reactivity:No Data Available.

Chemical Stability:Stable under Recommended Storage Conditions.

Possibility of Hazardous Reactions:No Data Available

Conditions to Avoid:

Heat, Flames and Sparks. Extremes of Temperature and Direct Sunlight.

Incompatible Materials:Strong Oxidizing Agents, Strong Bases

Hazardous Decomposition Products:

Other Decomposition Products - No Data Available

Section 11: Toxicological Information

Information on toxicological effects

Acute toxicity:LD50 Intraperitoneal - mouse - 800 mg/kg

Skin corrosion/irritation:

Skin - rabbit

Result: Skin irritation - 24 h

Serious eye damage/eye irritation:No data available.

Respiratory or skin sensitization:No data available.

Germ cell mutagenicity:No data available

Carcinogenicity

IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC.

Specific target organ toxicity - single exposure:Inhalation - May cause respiratory irritation.

Specific target organ toxicity - repeated exposure:No data available

Aspiration hazard:No data available.

Section 12: Ecological Information

Toxicity:No data available.

Persistence and degradability:No data available.

Bioaccumulative potential:No data available.

Mobility in soil:No data available.

Results of PBT and vPvB assessment:

PBT/vPvB assessment not available as chemical safety assessment not required/not conducted.

Other adverse effects:No data available.

Section 13: Disposal Considerations

Waste treatment methods

Product:

Burn in a chemical incinerator equipped with an afterburner and scrubber but exert extra care in igniting as this material is highly flammable. Offer surplus and non-recyclable solutions to a licensed disposal company.

Contaminated packaging:Dispose of as unused product.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name  : ETHYL ISOBUTYRATE

UN N°                            : 2385

H.I. nr: -

ADR – Class                  : 3

Labelling – Transport  : 3 : Flammable liquid.

ADR – Group                : II

Section 14: Transport Information (Continued)

Sea transport (IMDG) [English only]

Proper shipping name                 : ETHYL ISOBUTYRATE

UN N°                                            : 2385

IMO-IMDG - Class or division   : 3 : Flammable liquid.

IMO-IMDG - Packing group      : II

Air transport (ICAO-IATA) [English only]

Proper shipping name                  : ETHYL ISOBUTYRATE

UN N°                                             : 2385

IATA - Class or division               : 3 : Flammable liquid.

IATA - Packing group                  : II

Section 15: Other Regulatory Information

This safety datasheet complies with the requirements of Regulation (EC) No. 1907/2006.

Safety, health and environmental regulations/legislation specific for the substance or mixture:

No data available.

Chemical Safety Assessment:

For this product a chemical safety assessment was not carried out

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.