MATERIAL SAFETY DATA SHEET

EUPAROL

(For Microscopy)

MSDS CAS: -

Section 1: Chemical Product and Company Identification

Section 1: Chemical Product

Product Name:EUPAROL

CAS#: Not Available.

Synonym:Not Available.

Chemical Name: Euparol

Chemical Formula:Not Available.

Brand: OXFORD

Details Of The Supplier Of The Safety Data Sheet :

Company identification:   OXFORD LAB FINE CHEM LLP

Unit. No. 12, 1st Floor, Neminath Industrial Estate No. 6,

Navghar, Vasai (East). Palghar - 401 210.

Mumbai, Maharashtra, INDIA.

Tel: 91-250-2390989

Tel/Fax:  91-250-2390032

Section 2: Composition and Information on Ingredients

Composition:

| Substance name | Contents | CAS No | EC No | Annex No | REACH Ref. | Classification |
| --- | --- | --- | --- | --- | --- | --- |
| Xylene | > 50 < 75 % | 1330-20-7 | 215-535-7 | 601-022-00-9 | ---- | R10 Xn; R20/21 Xi; R38 ---------------------------------- Flam. Liq. 3 _ H226 Acute Tox. 4 (skin) _ H312Acute Tox. 4 (inhal) _ H332 Skin Irrit. 2 _ H315STOT SE 3 _ H335 |

Section 2: Composition and Information on Ingredients

| Substance name | Contents | CAS No | EC No | Annex No | REACH Ref. | Classification |
| --- | --- | --- | --- | --- | --- | --- |
| Dibutyl phthalate | > 1 < 5 % | 84-74-2 | 201-557-4 | 607-318-00-4 | ---- | Repr. Cat. 2; R61 Repr. Cat. 3; R62 N; R50 ---------------------------------- Repr. 1B _ H360D Repr. 2 _ H361f Aquatic Acute 1 _ H400 |

Section 3: Hazards Identification

Classification of the substance or mixture

Classification EC 67/548 or EC 1999/45

Classification:Repr. Cat. 2; R61 Xn; R20/21 Xi; R38

Hazard Class and Category Code(s), Regulation (EC) No 1272/2008 (CLP)

Health hazards:Reproductive toxicity - Unborn Child - Category 1A - Danger (CLP: Repr. 1A)

H360Reproductive toxicity - Fertility - Category 2 - Warning (CLP: Repr. 2) H361

Specific Target Organ Toxicity - Single exposure - Respiratory tract irritation –

Category 3 - Warning (CLP: STOT SE 3) H335

Skin irritation - Category 2 - Warning (CLP: Skin Corr. 2) H315

Acute toxicity, Inhalation - Category 4 - Warning (CLP: Acute Tox. 4) H332

Acute toxicity, dermal - Category 4 - Warning (CLP: Acute Tox. 4) H312

Label elements

Labelling EC 67/548 or EC 1999/45

Phrase(s):

R20/21: Harmful by inhalation and in contact with skin.

R38: Irritating to skin.

R61: May cause harm to the unborn child.

S Phrase(s):

S4: Keep away from living quarters.

S24: Avoid contact with skin.

S28: After contact with skin, wash immediately with

S35: This material and its container must be disposed of in a safe way.

S36/37: Wear suitable protective clothing and gloves.

S38: In case of insufficient ventilation, wear suitable respiratory equipment.

Section 3: Hazards Identification (Continued)

S45: In case of accident or if you feel unwell, seek medical advice immediately (show the label when possible).

S53: Avoid exposure - obtain special instructions before use.

S59: Refer to manufacturer/supplier for information on recovery/recycling.

Contains:Xylene - Dibutyl phthalate

Labelling Regulation EC 1272/2008 (CLP)

Hazard statements:

H360D: May damage the unborn child.

H361f: Suspected of damaging fertility.

H312: Harmful in contact with skin.

H315: Causes skin irritation.

H332: Harmful if inhaled.

H335: May cause respiratory irritation.

Precautionary statements

General: P201: Obtain special instructions before use.

Prevention:

P280: Wear protective gloves, protective clothing, eye protection, face protection.

P271: Use only outdoors or in a well-ventilated area.

P261: Avoid breathing dust, fume, gas, mist, vapours, and spray.

P264: Wash thoroughly after handling.

Response:

P302+P352: IF ON SKIN: Wash with plenty of soap and water.

P308+P313: If exposed or concerned: get medical advice.

P332+P313: If skin irritation occurs: Get medical advice.

P304+P340: IF INHALED: Remove to fresh air and keep at rest in a position comfortable for breathing.

P363: Wash contaminated clothing before reuse.

Storage:

P403+P233: Store in well-ventilated place. Keep container tightly closed.

P405: Store locked up.

Disposal considerations:

P501: Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Section 3: Hazards Identification (Continued)

Special labelling:

Restricted to professional users. (Substances which appear in Part 3 of Annex VI to Regulation (EC) No 1272/2008)

Contains:Xylene - Dibutyl phthalate

Other hazards:

The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Section 4: First Aid Measures

Description of first aid measures

Inhalation:

Call a POISON CENTER or doctor if you feel unwell. Remove to fresh air and keep at rest in a position comfortable for breathing.

Skin contact:

Immediately call a POISON CENTER or doctor. Specific treatment (see on this label). If skin irritation occurs: Wash with plenty of soap and water. Wash contaminated clothing before reuse. Get medical advice.

Eye contact:

Rinse immediately with plenty of water. Obtain medical attention if pain, blinking or redness persist.

Ingestion:

Obtain emergency medical attention. Rinse mouth. Do NOT induce vomiting.

Most important symptoms and effects, both acute and delayed

Symptoms relating to use:

May damage the unborn child.

Repeated exposure to this material can result in absorption through skin causing significant health hazard.

Suspected of damaging fertility.

Causes skin irritation. May cause respiratory irritation.

Danger of serious damage to health by prolonged exposure through inhalation.

Indication of any immediate medical attention and special treatment needed

General information:Never give anything by mouth to an unconscious person. If exposed or concerned: get medical advice.

Section 5: Fire and Explosion Data

Extinguishing media

Suitable extinguishing media: Foam. Dry powder. Carbon dioxide. Water spray. Sand.

Unsuitable extinguishing media: Do not use a heavy water stream.

Surrounding fires: Use water spray or fog for cooling exposed containers.

Special hazards arising from the substance or mixture

Hazardous combustion products: Under fire conditions, hazardous fumes will be present.

Advice for fire-fighters

Protection against fire: Do not enter fire area without proper protective equipment, including respiratory protection.

Special procedures: Exercise caution when fighting any chemical fire. Avoid (reject) fire-fighting water to enter environment.

Section 6: Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

For emergency responders: Equip cleanup crew with proper protection. Ventilate area.

For non-emergency personnel: Evacuate unnecessary personnel.

Environmental precautions:

Prevent entry to sewers and public waters. Notify authorities if product enters sewers or public waters.

Methods and material for containment and cleaning up

Soak up spills with inert solids, such as clay or diatomaceous earth as soon as possible. Store away from other materials. Collect spillage.

Section 7: Handling and Storage

Precautions for safe handling

Handling:Wash thoroughly after handling. Obtain special instructions before use. Avoid breathing dust, fume, gas, mist, vapours, and spray. Use only outdoors or in a well ventilated area.

Technical protective measures:Provide good ventilation in process area to prevent formation of vapour.

Special precautions:Avoid breathing dust, fume, gas, mist, vapours, and spray.

Section 7: Handling and Storage (Continued)

Conditions for safe storage, including any incompatibilities

Storage: Keep only in the original container in a cool, well ventilated place. Keep container tightly closed.

Storage - away from: Strong bases. Strong acids. Sources of ignition. Direct sunlight.

Specific end use(s):None.

Section 8: Exposure Controls/Personal Protection

Exposure controls

Personal protection

Personal protection: Avoid all unnecessary exposure.

Respiratory protection: In case of insufficient ventilation, wear suitable respiratory equipment.

Hand protection: Wear protective gloves.

Skin protection: Wear suitable protective clothing.

Eye protection: Chemical goggles or safety glasses.

Others: When using, do not eat, drink or smoke.

Section 9: Physical and Chemical Properties

Information on basic physical and chemical properties

Physical state at 20 °C			: Liquid.

Colour					: Yellow liquid.

Odour					: irritating odour.

Odour threshold				: No data available.

pH value					: No data available.

Melting point [°C]			: No data available.

Decomposition point [°C]		: Not Available.

Critical temperature [°C]		: No data available.

Auto-ignition temperature [°C]	: No data available.

Flammability (solid, gas)		: Not Available.

Flash point [°C]				: No data available.

Boiling point [°C]				: Not Available.

Initial boiling point [°C]		: No data available.

Final boiling point [°C]			: No data available.

Section 9: Physical and Chemical Properties (Continued)

Evaporation rate				: Not Available.

Vapour pressure [20°C]			: No data available.

Vapour pressure mm/Hg		: Not Available.

Vapour density				: Not Available.

Density [g/cm3]				: Not Available.

Relative density, gas (air=1)		: Not Available.

Relative density, liquid (water=1)	: Not Available.

Solubility in water [% weight]	: Insoluble in water

Solubility in water			: No data available.

Log Pow octanol / water at 20°C	: No data available.

Solubility					: Not Available.

Viscosity at 40°C [mm2/s]		: No data available.

Section 10: Stability and Reactivity Data

Reactivity:Not established.

Chemical stability:Stable under recommended storage conditions.

Possibility of hazardous reactions:Not established.

Conditions to avoid:Direct sunlight. Extremely high or low temperatures.

Incompatible materials:Strong acids. Strong bases.

Hazardous decomposition products:Fumes. Carbon monoxide. Carbon dioxide.

Section 11: Toxicological Information

Information on toxicological effects:

Toxicity information: The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations.

Section 11: Toxicological Information (Continued)

Acute toxicity

Inhalation: Harmful if inhaled.

Dermal: Harmful in contact with skin.

Ingestion: Based on available data, the classification criteria are not met.

Corrosion: Based on available data, the classification criteria are not met.

Irritation: Causes skin irritation.May cause respiratory irritation.

Sensitization: Based on available data, the classification criteria are not met.

Mutagenicity: Based on available data, the classification criteria are not met.

Carcinogenicity: Based on available data, the classification criteria are not met.

Toxic for reproduction: May damage the unborn child. Suspected of damaging fertility.

STOT-single exposure: Based on available data, the classification criteria are not met.

STOT-repeated exposure: Based on available data, the classification criteria are not met.

Aspiration hazard: Based on available data, the classification criteria are not met.

Section 12: Ecological Information

Toxicity information:

The product has been not fully tested. The calculated risk has been done under the requirements of the EU regulations.

Persistence - degradability:Biodegradable.

Bioaccumulative potential:Not established.

Mobility in soil:Not established.

Results of PBT and vPvB assessment:

The substance does not fulfil the criteria to be identified as PBT substance or vPvB substance according to Annex XIII of Regulation REACH.

Environmental precautions:Avoid release to the environment.

Section 13: Disposal Considerations

Waste treatment methods

General:Avoid release to the environment. Dispose in a safe manner in accordance with local/national regulations. Dispose of this material and its container to hazardous or special waste collection point, in accordance with local, regional, national and/or international regulation.

Section 14: Transport Information

Land transport (ADR-RID)

Proper shipping name: FLAMMABLE LIQUID, N.O.S.

UN N°: 1993

H.I. nr: 30

ADR - Class: 3

Labelling - Transport: 3: Flammable liquid.

ADR - Group: III

Sea transport (IMDG) [English only]

Proper shipping name: FLAMMABLE LIQUID, N.O.S.

UN N°: 1993

IMO-IMDG - Class or division: 3: Flammable liquid.

IMO-IMDG - Packing group: III

Air transport (ICAO-IATA) [English only]

Proper shipping name: FLAMMABLE LIQUID, N.O.S.

UN N°: 1993

IATA - Class or division: 3: Flammable liquid.

IATA - Packing group: III

Section 15: Other Regulatory Information

Safety, health and environmental regulations/legislation specific for the substance or mixture

Safety, health and environmental regulations/legislation specific for the substance or mixture:Ensure all national/local regulations are observed.

Section 15: Other Regulatory Information (Continued)

REACH Restrictions - Annex XVII

With the exception of those listed below:

The components of this product are not subject to restrictions.

Dibutyl phthalate: Annex XVII

REACH Authorisation - Annex XIV

With the exception of those listed below:

The components of this product are not subject to authorization.

Dibutyl phthalate: Annex XIV

Dibutyl phthalate: Annex XIV

Chemical Safety Assessment:It has not been carried out.

Section 16 - Additional Information

References: Not available.

Other Special Considerations: Not available.

Disclaimer:

**************************************************************************************************************************

The information contained herein in good faith but makes no representations as to its comprehensiveness or accuracy. This document is intended only as a guide to the appropriate precautionary handling of the material by a properly trained person using this product. Individuals receiving the information must exercise their independent judgment in determining its appropriateness for a particular purpose.

Oxford Lab Fine ChemLLP makes no representations or warranties, either express or implied, including without limitation any warranties of merchantability, fitness for a particular purpose with respect to the information set forth herein or the product to which the information refers. Accordingly, Oxford Lab Fine Chem LLP will not be responsible for damages resulting from use of or reliance upon this information.